VoltStartEV

End-to-End Test Cases

Version 1.0  |  June 2026  |  Frontend · Backend · OCPP · SteVe

# Test Environment

| Component | Details |
| --- | --- |
| Backend URL | http://136.113.7.146:3000 |
| SteVe URL | http://136.113.7.146:8080 |
| OCPP WS | ws://136.113.7.146:8080/steve/websocket/CentralSystemService/{chargeBoxId} |
| Test User | qatest001 / QATest123! (userId=33, role=fleet_admin) |
| Test User 2 | qatest002 / QATest123! (userId=30, role=customer) |
| Test OCPP Tag | QATEST001 (system tag), A1A2A3A4 (external RFID) |
| Simulator | SAP e-mobility-charging-stations-simulator (CLI) |
| Test Chargers | CS-HPC350K-00001 (DC 350kW), CS-AC7K-00001 (AC 7kW), CS-SCHUKO3K-00001 (3-pin) |
| Mobile | Expo Go SDK 53 · React Native · Android/iOS |
| DB | MySQL 8.0 stevedb + voltstartev_db |

TIP Run all SAP CLI commands from the simulator directory. Prefix with the station hashId obtained from: evse-cli station list

# Priority Legend

| Priority | Label | Description |
| --- | --- | --- |
| P0 | Critical | App unusable if fails. Must pass before any release. |
| P1 | High | Core user flow broken. Must pass before QA sign-off. |
| P2 | Medium | Important feature degraded. Fix before release if possible. |
| P3 | Low | Edge case or cosmetic. Track and fix in next sprint. |

# Module 1 — Authentication

## 1.1 Registration

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-AUTH-001 | New user registration with valid data | App installed, no existing account | POST /api/users/register returns 201, JWT token in response, user row in users table, OCPP tag auto-generated in user_tags and stevedb.ocpp_tag | curl -X POST .../register -d '{"username":"newuser","email":"new@test.com","password":"Test1234!"}' | P0 |
| TC-AUTH-002 | Registration with duplicate email | TC-AUTH-001 completed | API returns 409 Conflict, no duplicate user created, app shows "Email already registered" alert | Same request as TC-AUTH-001 | P0 |
| TC-AUTH-003 | Registration with weak password | None | API returns 400, error mentions password requirements, no user created | POST /register with password="abc" | P1 |
| TC-AUTH-004 | OCPP tag auto-assigned on registration | TC-AUTH-001 completed | SELECT ocpp_tag_id FROM user_tags WHERE app_user_id=NEW_ID returns VSE-{id}-{xxxx} format, same tag exists in stevedb.ocpp_tag | mysql: SELECT * FROM user_tags WHERE app_user_id=NEW_ID | P0 |

## 1.2 Login

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-AUTH-010 | Login with valid credentials | User exists in DB | POST /api/users/login returns 200, token in response, token stored in AsyncStorage, app navigates to MapScreen | curl -X POST .../login -d '{"username":"qatest001","password":"QATest123!"}' | P0 |
| TC-AUTH-011 | Login with wrong password | User exists | Returns 401, app shows "Invalid credentials" alert, no token stored | POST /login with wrong password | P0 |
| TC-AUTH-012 | JWT token expiry handling | Login completed, manually expire token in DB or wait | API returns 401 with "expired" message, app clears AsyncStorage, navigates to Login screen | Modify JWT_SECRET temporarily then call any authenticated endpoint | P1 |
| TC-AUTH-013 | Login persists across app restart | User logged in, close and reopen app | App auto-navigates to MapScreen without showing login, token loaded from AsyncStorage | Close Expo Go, reopen | P1 |
| TC-AUTH-014 | Logout clears all state | User logged in | All AsyncStorage keys cleared, authStore reset, WebSocket disconnected, app shows Login screen | Tap Logout in Profile → confirm | P1 |

## 1.3 WebSocket Authentication

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-WS-001 | WebSocket connects with token in URL | User logged in, app open | Logs show "WS authenticated, userId: X", backend logs show "WebSocket connected: user_X" | Check mobile logs after login | P0 |
| TC-WS-002 | WebSocket rejected with invalid token | Manually corrupt token | Backend closes WS with code 4001, mobile logs show "Authentication required", reconnect not attempted | Modify token in AsyncStorage to invalid value | P1 |
| TC-WS-003 | WebSocket reconnects after network drop | Session active, simulate network loss | After network restored, WS reconnects automatically, telemetry resumes within 30s, exponential backoff visible in logs | Toggle airplane mode on device | P1 |

# Module 2 — Map & Charger Discovery

## 2.1 Map Loading

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-MAP-001 | Map loads with all 12 chargers visible | Logged in, location permission granted | GET /api/chargers returns 12 chargers, all markers visible on map, charger count badge shows "12/12 chargers" | curl -H "Authorization: Bearer $TOKEN" .../api/chargers ¦ python3 -m json.tool | P0 |
| TC-MAP-002 | Charger config cache hit on second load | TC-MAP-001 run once | Second GET /api/chargers shows "CHARGER CONFIG CACHE HIT" and "PRICING CACHE HIT" in backend logs, response < 200ms | Watch backend logs during second charger load | P1 |
| TC-MAP-003 | Map centers on user location | GPS enabled, location permission granted | Blue dot appears at device location, map animates to user position within 2 seconds of opening | Allow location permission, check map | P1 |
| TC-MAP-004 | Charger marker colors by status | Chargers in various states | Available=green, Busy/Charging=amber/blue, Faulted=red, Offline=gray, Reserved=purple | evse-cli ocpp status-notification --connector-id 1 --status Charging {hash} | P1 |
| TC-MAP-005 | Map markers update without 2-min poll | MapScreen open, charger Available | After evse-cli status change, map marker updates within 3 seconds via WebSocket, not waiting for poll | evse-cli ocpp status-notification --connector-id 1 --status Charging {hash} | P0 |

## 2.2 Charger Modal

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-MAP-010 | Tap marker opens charger detail modal | Map loaded with chargers | Modal opens with charger name, status, connector count, pricing, "Start Charging" button visible | Tap any green marker on map | P0 |
| TC-MAP-011 | Modal shows live connector statuses | Charger modal open | Each connector shows current OCPP status (Available/Charging/etc) with correct color | Open modal while evse-cli sends various statuses | P1 |
| TC-MAP-012 | Pricing estimate shows in modal | Charger has active pricing in charger_pricing table | Modal shows "₹X.XX/kWh" or tiered rates, pricing-estimate endpoint called with user vehicle data | Check GET .../api/chargers/{id}/pricing-estimate response | P1 |
| TC-MAP-013 | Modal updates when connector status changes | Modal open, charger Available | When evse-cli sends Charging status, modal connector card updates from green to blue in real time without closing modal | evse-cli ocpp status-notification --connector-id 1 --status Charging {hash} | P0 |
| TC-MAP-014 | Offline charger shows correct state | Stop the simulator for a charger | Charger shows gray "Offline" marker after heartbeat timeout (5 min), modal shows "Offline" status | evse-cli connection close {hash} then wait 5 min | P2 |

## 2.3 Filters

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-FILTER-001 | Filter by Available only | Multiple chargers with mixed statuses | Only Available chargers shown, count badge updates, Busy/Offline markers hidden | Set availability filter to "Available", verify count decreases | P1 |
| TC-FILTER-002 | Filter by min power 50kW | Chargers with various max_power_w values | Only DC fast chargers (50kW+) shown, AC 7kW chargers hidden | Set minPower=50 filter | P1 |
| TC-FILTER-003 | Filter by connector type CCS2 | charger_capabilities table populated | Only chargers with CCS2 connectors shown | Set connectorType=CCS2 filter | P2 |
| TC-FILTER-004 | Filter by vehicle type 2W | charger_capabilities with 2W entries | Only Bharat AC/DC and 3-Pin chargers shown | Set vehicleType=2W filter | P2 |
| TC-FILTER-005 | Reset filters shows all chargers | Filters applied in TC-FILTER-001 through 004 | All 12 chargers visible, count badge shows 12/12 | Tap Reset Filters | P1 |
| TC-FILTER-006 | Price filter uses max tier for tiered pricing | HPC350K has tiered pricing up to ₹28/kWh | Setting maxPrice=₹20 hides HPC350K (max tier ₹28), does not show it at base rate ₹16 | Set maxPrice=20, verify HPC350K hidden | P1 |

# Module 3 — Charging Session

## 3.1 Session Start

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-SESSION-001 | Start charging via app (happy path) | Wallet >= ₹50, valid tag, charger Available | POST /api/charging/start returns 200, SteVe receives RemoteStartTransaction, evse-cli starts transaction, session row created in charging_sessions, SessionScreen appears | evse-cli atg start {hash} --connector-ids 1 (to auto-respond) | P0 |
| TC-SESSION-002 | Session blocked when wallet < ₹50 | User wallet balance = ₹30 | POST /api/charging/start returns 402 INSUFFICIENT_BALANCE, app shows "Add Money" alert with Wallet navigation | Set balance: UPDATE wallets SET balance=30 WHERE user_id=33 | P0 |
| TC-SESSION-003 | Session blocked when wallet = ₹0 | User wallet balance = ₹0 | Same as TC-SESSION-002, balance check fires before SteVe call | UPDATE wallets SET balance=0 WHERE user_id=33 | P0 |
| TC-SESSION-004 | Correct idTag sent to SteVe | User has primary tag A1A2A3A4 set | Backend logs show idTag=A1A2A3A4 in StartTransaction, not QATEST001 | Check backend logs: "Charging session start requested" | P0 |
| TC-SESSION-005 | Session start with expired tag blocked | Set expiry_date in past for QATEST001 in stevedb.ocpp_tag | Returns 403 "Your RFID tag has expired", no SteVe call made | UPDATE stevedb.ocpp_tag SET expiry_date="2020-01-01" WHERE id_tag="QATEST001" | P1 |
| TC-SESSION-006 | Pricing stored at session creation | Session started on CS-HPC350K-00001 | charging_sessions.rate_per_kwh and session_fee populated at INSERT, not null | SELECT rate_per_kwh, session_fee FROM charging_sessions WHERE steve_transaction_pk=LATEST | P0 |
| TC-SESSION-007 | Session fee charged only once | HPC350K has ₹100 session fee | Session fee appears in first meter value cost delta only, subsequent meter values charge only energy rate | Watch backend logs: "isFirstChunk" flag in cost calculation | P0 |

## 3.2 Live Telemetry

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-TEL-001 | Telemetry updates in SessionScreen every 5s | Active charging session | SessionScreen shows updating kWh, cost, power, current, voltage every ~5 seconds via WebSocket | evse-cli ocpp meter-values --connector-id 1 {hash} | P0 |
| TC-TEL-002 | Cost calculation correct for per_kwh model | Session on AC7K (₹7.50/kWh) | After 1 kWh delivered, costSoFar = ₹7.50 ± ₹0.01 | Send MeterValues with 1000 Wh, check telemetry:update.costSoFar | P0 |
| TC-TEL-003 | Tiered pricing applies correct rate per interval | Session on HPC350K (tiered ₹16/22/28/kWh) | When powerW=80000W, rate=₹22/kWh. When powerW=200000W, rate=₹28/kWh | Send MeterValues with powerW=80000 then 200000, check backend logs "ratePerKwh" | P0 |
| TC-TEL-004 | Cost is monotonically increasing | Active session, multiple meter values | costSoFar never decreases between consecutive telemetry updates (monotonic guard) | Send 10 meter values, verify each costSoFar >= previous | P0 |
| TC-TEL-005 | Backward meter value ignored | Active session, meterWh > 0 | If MeterValues reports lower Wh than previous, backend ignores it, logs "Ignoring backward meter", cost unchanged | Send MeterValues with meterWh lower than previous reading | P1 |
| TC-TEL-006 | SOC shown in SessionScreen when available | Charger sends SoC measurand | Battery percentage visible in SessionScreen stats row | evse-cli ocpp meter-values -p '{"meterValue":[{"sampledValue":[{"measurand":"SoC","value":"65","unit":"Percent"}]}]}' {hash} | P2 |
| TC-TEL-007 | Temperature shown when above threshold | Charger sends Temperature measurand > 45C | Temperature displayed in amber, > 60C shows red warning icon | Send MeterValues with Temperature=55, then 65 | P2 |
| TC-TEL-008 | Telemetry updates HistoryScreen live cost | HistoryScreen open, active session | Active session card shows updating cost in blue, not zero | Open History tab while session active | P0 |
| TC-TEL-009 | History pull-to-refresh preserves live cost | TC-TEL-008 in progress | After pull-to-refresh, live cost does not reset to zero — shows last_cost from DB as seed | Pull down to refresh on HistoryScreen during active session | P0 |

## 3.3 Session Stop

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-STOP-001 | Stop session via app (happy path) | Active session | POST /api/charging/stop → RemoteStopTransaction sent to SteVe → StopTransaction webhook received → session.status=completed, total_cost calculated, wallet deducted | Tap Stop in SessionScreen | P0 |
| TC-STOP-002 | Wallet correctly deducted on stop | Active session, wallet=₹200, session cost=₹45 | wallet.balance = ₹155 after session_completed webhook processed | SELECT balance FROM wallets WHERE user_id=33 after stop | P0 |
| TC-STOP-003 | Session appears in History after stop | TC-STOP-001 completed | New completed session visible in HistoryScreen with correct kWh and cost | Navigate to History tab after stopping | P0 |
| TC-STOP-004 | Map marker returns to Available after stop | TC-STOP-001 completed | Map marker turns green within 5 seconds of StopTransaction — via WebSocket not poll | Watch map after tapping Stop | P0 |
| TC-STOP-005 | Stop button debounce prevents double-stop | Active session | Tapping Stop multiple times quickly only fires one RemoteStopTransaction | Tap Stop rapidly 3 times, check backend logs for single stop request | P1 |
| TC-STOP-006 | Session stopped via charger (local stop) | Active session, physically stop at charger | SteVe receives StopTransaction with reason=Local, webhook fires, session completes, wallet deducted correctly | evse-cli transaction stop --transaction-id {txId} {hash} | P1 |
| TC-STOP-007 | Stop reason EmergencyStop handled | Active session | StopTransaction with reason=EmergencyStop creates completed session, user notified | evse-cli transaction stop -p '{"reason":"EmergencyStop"}' --transaction-id {txId} {hash} | P2 |

## 3.4 Auto-Stop

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-AUTO-001 | SOC auto-stop fires at target SOC | Session active, user target SOC=80%, real SOC < 80% | When SOC reaches 80%, RemoteStopTransaction sent, session_completed, push notification received | evse-cli ocpp meter-values -p with SoC values 70,75,79,80 | P0 |
| TC-AUTO-002 | SOC auto-stop not triggered by simulator noise | Random SOC values (0-100% each interval) | Auto-stop not triggered when SOC jump > 20% per interval (noise guard) | Send SoC: 20,90,10,80,5 — verify no stop fires | P0 |
| TC-AUTO-003 | SOC auto-stop fires only once | Session active, target SOC=80% | RemoteStopTransaction sent exactly once even if multiple meter values arrive simultaneously at 80% | Send 3 rapid meter values all with SoC=80, check backend logs for single stop | P0 |
| TC-AUTO-004 | Low balance auto-stop fires at ₹5 remaining | Wallet=₹20, session on HPC350K (₹28/kWh) | When costSoFar reaches ₹15 (₹5 threshold), RemoteStop sent, balance_critical WebSocket event received | Send MeterValues accumulating to ₹15 cost | P0 |
| TC-AUTO-005 | Low balance auto-stop fires at exactly ₹0 | Wallet=₹0 | Immediate RemoteStop on first meter value, session never accumulates cost beyond balance | UPDATE wallets SET balance=0, then start session and send any MeterValues | P0 |
| TC-AUTO-006 | Balance_critical alert shown in app | TC-AUTO-004 in progress | App shows "Wallet Balance Critical" alert with "Add Money" button navigating to Wallet tab | Watch app during low balance auto-stop | P1 |
| TC-AUTO-007 | SOC target update mid-session works | Active session, target SOC=80% | User changes target to 95% in Profile → PUT .../target-soc → WebSocket soc_target_updated event → auto-stop now fires at 95% | Change target SOC in Profile while session active, send SoC=82 (should not stop), then SoC=96 (should stop) | P1 |

# Module 4 — Wallet

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-WALLET-001 | Wallet balance displayed correctly | User has wallet with balance | GET /api/wallet returns correct balance, WalletScreen shows same value | curl -H "Authorization: Bearer $TOKEN" .../api/wallet | P0 |
| TC-WALLET-002 | Load wallet via Razorpay WebView | User on WalletScreen | Razorpay WebView opens, payment succeeds, balance increases by loaded amount, transaction appears in wallet_transactions | Tap "Add Money" → complete test payment with Razorpay test card | P0 |
| TC-WALLET-003 | Razorpay webhook signature verified | Valid Razorpay webhook with correct secret | Wallet credited only when HMAC-SHA256 signature matches, rejected otherwise | Send webhook with correct then incorrect signature | P0 |
| TC-WALLET-004 | Transaction history shows all types | Multiple transactions: credit, debit, refund | WalletScreen shows all transactions with correct icons, amounts, dates | Complete a session (debit) and load money (credit) | P1 |
| TC-WALLET-005 | Live balance shown during session | Active charging session, SessionScreen open | Wallet balance card shows decreasing balance in real time (refreshed every 30s) | Watch SessionScreen during active session | P1 |
| TC-WALLET-006 | Duplicate payment order rejected | Valid payment order created | Replaying the same Razorpay webhook does not credit wallet twice (idempotency check) | POST same webhook payload twice | P0 |

# Module 5 — RFID & Charging Tags

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-RFID-001 | Add external RFID card | User on Profile → RFID section | POST /api/users/me/rfid with valid UID → row in user_tags, row in stevedb.ocpp_tag, row in stevedb.user_ocpp_tag, card visible in RFID section | Tap + Add RFID → enter A1A2A3A4 | P0 |
| TC-RFID-002 | Set external RFID as primary | TC-RFID-001 completed | PUT .../primary → is_primary=1 for new card, is_primary=0 for previous primary, OCPP tag in Profile header updates | Tap Set Primary on new card | P0 |
| TC-RFID-003 | Remove RFID card (soft delete) | External RFID card exists, is not primary | DELETE .../rfid/{id} → user_tags.is_active=0, system tag promoted to primary, SteVe tag blocked (expiry=2000-01-01) | Tap Remove on RFID card | P0 |
| TC-RFID-004 | Cannot remove system tag | System tag (App Tag) visible in RFID section | Tap Remove on system tag → alert "Cannot Remove — used as fallback", no API call made | Attempt to remove App Tag | P1 |
| TC-RFID-005 | RFID tag validates for session start | External RFID A1A2A3A4 is primary | POST /api/charging/start with idTag=A1A2A3A4 → validateTagForUser checks user_tags table → passes | Start session with external RFID as primary tag | P0 |
| TC-RFID-006 | Profile OCPP tag updates after set primary | Two cards: QATEST001 and A1A2A3A4 | After setting A1A2A3A4 as primary, Profile Account Info row shows A1A2A3A4 not QATEST001 | Change primary, check Profile header | P1 |
| TC-RFID-007 | Steve sync queue retries on failure | SteVe down, RFID deleted | Deletion queued in steve_sync_queue table, processed when SteVe comes back, exponential backoff in logs | Stop SteVe, delete RFID, restart SteVe, verify sync completes | P1 |

# Module 6 — Reservations

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-RES-001 | Reserve a connector | Charger Available, no active reservation | POST /api/reservations → SteVe ReserveNow OCPP sent, connector shows Reserved in modal, app_reservations row created | Tap Reserve in charger modal | P1 |
| TC-RES-002 | Cannot start session on connector reserved by another user | Connector reserved by qatest002 | Attempt to start session as qatest001 → blocked, connector shows Reserved badge | Reserve as user2, attempt start as user1 | P1 |
| TC-RES-003 | Cancel reservation | Active reservation exists | DELETE /api/reservations/{id} → CancelReservation sent to SteVe → connector returns to Available | Tap Cancel Reservation in modal | P1 |
| TC-RES-004 | Active reservation shown in modal on open | User has active reservation | Opening charger modal shows "Your Reservation" badge on reserved connector | Have reservation, open modal | P2 |

# Module 7 — Vehicles & Profile

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-VEH-001 | Add vehicle | User on Profile | POST /api/users/me/vehicles → vehicle row in user_vehicles, appears in Profile vehicles section | Add vehicle: MG ZS EV, 50.3 kWh | P1 |
| TC-VEH-002 | Set primary vehicle | Two vehicles exist | PUT .../primary → is_primary=1 for selected, 0 for others | Tap Set Primary on second vehicle | P1 |
| TC-VEH-003 | Target SOC control always visible | Vehicle exists | TargetSocControl renders below primary vehicle card, shows current SOC target, presets visible | Check Profile screen | P1 |
| TC-VEH-004 | Target SOC update persists | Vehicle with target_soc=80 | Change to 90% via TargetSocControl → PUT .../target-soc → DB updated → pricing estimate uses new SOC | Change target SOC, verify DB: SELECT target_soc FROM user_vehicles WHERE is_primary=1 | P1 |
| TC-VEH-005 | Target SOC update during active session | Active charging session, target=80% | Change target to 95% → PUT .../target-soc → soc_target_updated WebSocket event received → SessionScreen target updates | Change target during active session, watch SessionScreen | P1 |
| TC-VEH-006 | Delete vehicle (not primary) | Two vehicles, primary and secondary | DELETE .../vehicles/{id} → vehicle removed from DB, not visible in Profile | Tap Remove on non-primary vehicle | P1 |
| TC-VEH-007 | Cannot delete primary vehicle when it is the only vehicle | Only one vehicle, is_primary=1 | Deletion blocked with appropriate message | Attempt to remove only vehicle | P2 |

# Module 8 — Fleet Management

Fleet features are gated behind FLEET_ENABLED=true env flag and role=fleet_admin JWT claim.

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-FLEET-001 | Fleet section visible for fleet_admin | qatest001 has role=fleet_admin | FleetSection renders in Profile, "Create Fleet" or fleet details visible | Login as qatest001, check Profile | P1 |
| TC-FLEET-002 | Fleet section hidden for regular user | qatest002 has role=customer | FleetSection not rendered in Profile for customer role | Login as qatest002, check Profile | P1 |
| TC-FLEET-003 | Create fleet | fleet_admin user, no existing fleet | POST /api/fleet → fleet row in fleets table, user assigned as admin in fleet_members | Create fleet with name "Test Fleet" | P2 |
| TC-FLEET-004 | Fleet billing mode: fleet pays | Active fleet with billing_mode=fleet_pays | Session started by fleet member billed to fleet wallet, not personal wallet | Start session as fleet member | P2 |
| TC-FLEET-005 | Fleet monthly limit enforcement | Fleet has monthly_limit=₹1000, spent ₹999 | Next session start blocked when monthly limit reached | Set monthly_limit low and attempt session start | P2 |

# Module 9 — Push Notifications

Note: Push notifications require a real dev build (not Expo Go). Test with dev build or verify via backend logs.

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-PUSH-001 | Session started push notification | Push token registered, session starts | Push notification received: "⚡ Charging Started! {chargeBoxId} · Connector {connectorId}" | Start session, check device notifications | P1 |
| TC-PUSH-002 | Session completed push notification | Active session completes | Push notification received with final cost and energy | Stop session, check device notifications | P1 |
| TC-PUSH-003 | SOC target reached push notification | TC-AUTO-001 completed | Push notification: "Target Charge Reached — vehicle at X%" | Check device after SOC auto-stop | P1 |
| TC-PUSH-004 | Low balance push notification | TC-AUTO-004 completed | Push notification: "Low Balance — Stopping Charge ₹X.XX remaining" | Check device after balance auto-stop | P1 |
| TC-PUSH-005 | Push token persists across login | User logs out and back in | push_token column in users table updated on re-login, notifications still received | Logout, login, start session | P2 |

# Module 10 — OCPP 1.6J Compliance

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-OCPP-001 | BootNotification accepted | Charger reconnects | SteVe responds Accepted, registration_status=Accepted in stevedb, charger shows online in app within 2 min | evse-cli ocpp boot-notification {hash} | P0 |
| TC-OCPP-002 | Heartbeat keeps charger online | Charger connected | last_heartbeat_timestamp updates every 60s in stevedb.charge_box | evse-cli ocpp heartbeat {hash} (repeat) | P1 |
| TC-OCPP-003 | StatusNotification updates DB immediately | Charger changes state | connector_status row inserted in stevedb, WebSocket broadcast fired, app map updates within 3s | evse-cli ocpp status-notification --connector-id 1 --status Charging --error-code NoError {hash} | P0 |
| TC-OCPP-004 | RemoteStartTransaction response | Charger connected, Available | SteVe receives RemoteStart, returns Accepted, simulator starts transaction | POST /api/charging/start, watch SteVe logs | P0 |
| TC-OCPP-005 | RemoteStopTransaction response | Active transaction | SteVe receives RemoteStop, returns Accepted, simulator stops transaction | POST /api/charging/stop, watch SteVe logs | P0 |
| TC-OCPP-006 | SetChargingProfile accepted | Charger connected, operator role | POST /api/chargers/{id}/connectors/1/charging-profile → SteVe SetChargingProfile sent, charger limits power | curl -X POST .../charging-profile -d '{"maxPowerWatts":50000}' | P1 |
| TC-OCPP-007 | ClearChargingProfile restores default | Charging profile applied | DELETE /api/chargers/{id}/connectors/1/charging-profile → ClearChargingProfile sent, charger returns to full power | curl -X DELETE .../charging-profile | P1 |
| TC-OCPP-008 | ChangeAvailability Inoperative | Charger connected, operator role | PUT /api/chargers/{id}/availability with Inoperative → OCPP ChangeAvailability sent, connector shows Unavailable | curl -X PUT .../availability -d '{"type":"Inoperative","connectorId":1}' | P1 |
| TC-OCPP-009 | MeterValues with all measurands extracted | Active session, rich meter values | telemetry_extractor correctly parses Energy.Active.Import.Register, Power.Active.Import, Current.Import, Voltage, SoC, Temperature | Send full MeterValues payload with all measurands | P0 |
| TC-OCPP-010 | Authorization tag expiry checked | OCPP tag has past expiry_date | Session start returns 403 "tag expired", no RemoteStartTransaction sent | Set expiry in past for tag, attempt start | P1 |

# Module 11 — Performance & Reliability

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-PERF-001 | Two users charging simultaneously | Two sessions active (qatest001 + qatest002) | Telemetry isolated per user, no cost cross-contamination, both sessions update independently | Start sessions as both users on different chargers simultaneously | P0 |
| TC-PERF-002 | Charger list loads in < 500ms (cache hit) | Cache warmed from previous request | GET /api/chargers with cache hit responds in < 500ms | Second request after cache warms, check response time | P1 |
| TC-PERF-003 | No double-fetch on app load | App cold start | Only one GET /api/chargers and one GET /api/charging/session/active in logs within first 3 seconds | Check backend logs on app launch | P1 |
| TC-PERF-004 | Reconciliation fixes orphaned sessions | Active session, backend restarted mid-session | Reconciliation job (runs every 10 min) marks stale sessions as completed, final cost calculated from last_cost | pm2 restart voltstartev mid-session, wait 10 min | P1 |
| TC-PERF-005 | Pool exhaustion handled gracefully | Multiple concurrent users (5+) | No ETIMEDOUT errors, requests queued up to queueLimit, slow queries logged as warnings | Start 5 simultaneous API requests | P1 |
| TC-PERF-006 | DB atomic OTP verification (no race condition) | OTP verification endpoint | Simultaneous duplicate OTP submissions: only first succeeds, second returns "OTP already used" | Send same OTP twice within 100ms | P0 |
| TC-PERF-007 | Auto-stop fires once even with concurrent meter values | Active session approaching SOC target | Multiple meter values arriving simultaneously only trigger one RemoteStop | Send 3 rapid meter values all at target SOC | P0 |

# Module 12 — UI / UX

| TC ID | Test Case | Precondition | Expected Result | SAP CLI / API Call | Priority |
| --- | --- | --- | --- | --- | --- |
| TC-UI-001 | Icons render correctly on Android (MIUI) | Xiaomi/Redmi device | No emoji boxes, all BoltIcon/AppIcon components render as expected, no "Text strings must be in Text" error | Run on Xiaomi device, check Profile and Map screens | P1 |
| TC-UI-002 | Map renders with tiles (not white) | Expo Go SDK 53 | Map tiles visible, markers positioned correctly, user location dot shown | Open app on device with GPS | P0 |
| TC-UI-003 | History screen shows zero after pull-to-refresh | Active session in progress | Pull-to-refresh on HistoryScreen preserves liveCost (does not reset to zero) | Pull refresh during active session | P0 |
| TC-UI-004 | New session appears in History immediately | HistoryScreen open when session starts | session_started WebSocket event triggers fetchHistory, new active session card appears without manual refresh | Open History, start session on another tab | P0 |
| TC-UI-005 | Session screen navigated automatically on start | Session starts via app | App navigates to Session tab automatically when session_started WebSocket event received | Start session while on Map tab | P1 |
| TC-UI-006 | Require cycles warning suppressed | App running | Circular dependency between api.ts and authStore.ts resolved via lazy require, WARN disappears from logs | Check Expo logs for "Require cycle" warning | P2 |
| TC-UI-007 | expo-notifications deprecation warning | App running | shouldShowAlert replaced with shouldShowBanner + shouldShowList in notification handler | Check Expo logs for deprecation warning | P3 |
| TC-UI-008 | Connector type badge visible in modal | charger_capabilities table populated | Connector cards in modal show "CCS2", "Type2", "BharatAC" badges | Open charger modal, check connector cards | P2 |
| TC-UI-009 | SOC auto-stop alert shown on correct screen | Session auto-stops due to SOC | Alert appears regardless of which tab user is on when soc_target_reached event fires | Be on History tab when SOC auto-stop fires | P1 |
| TC-UI-010 | Wallet low balance warning during session | Balance < ₹50 during active session | SessionScreen balance card shown in amber with warning indicator | Set balance to ₹30 then start session | P1 |

# Appendix A — SAP CLI Quick Reference

Run all commands from the SAP simulator directory. Get {hash} from: evse-cli station list

| Action | SAP CLI Command |
| --- | --- |
| List all stations | evse-cli station list |
| Check simulator state | evse-cli simulator state |
| Set connector Available | evse-cli ocpp status-notification --connector-id 1 --error-code NoError --status Available {hash} |
| Set connector Charging | evse-cli ocpp status-notification --connector-id 1 --error-code NoError --status Charging {hash} |
| Set connector Faulted | evse-cli ocpp status-notification --connector-id 1 --error-code GroundFailure --status Faulted {hash} |
| Send Heartbeat | evse-cli ocpp heartbeat {hash} |
| Send BootNotification | evse-cli ocpp boot-notification {hash} |
| Start ATG (auto transactions) | evse-cli atg start {hash} --connector-ids 1 |
| Stop ATG | evse-cli atg stop {hash} |
| Manual transaction start | evse-cli transaction start --connector-id 1 --id-tag QATEST001 {hash} |
| Manual transaction stop | evse-cli transaction stop --transaction-id {txId} {hash} |
| Send MeterValues (custom) | evse-cli ocpp meter-values --connector-id 1 -p '{"meterValue":[{"sampledValue":[{"measurand":"Energy.Active.Import.Register","value":"1000","unit":"Wh"},{"measurand":"SoC","value":"75","unit":"Percent"}]}]}' {hash} |
| Send specific SOC value | evse-cli ocpp meter-values --connector-id 1 -p '{"meterValue":[{"sampledValue":[{"measurand":"SoC","value":"80","unit":"Percent"}]}]}' {hash} |
| Open WebSocket connection | evse-cli connection open {hash} |
| Close WebSocket connection | evse-cli connection close {hash} |

# Appendix B — DB Verification Queries

| What to Check | Query |
| --- | --- |
| Active sessions | SELECT session_id, charge_box_id, status, last_cost, app_user_id FROM charging_sessions WHERE end_time IS NULL |
| Latest session cost | SELECT session_id, total_cost, last_cost, energy_kwh, stop_reason FROM charging_sessions ORDER BY start_time DESC LIMIT 5 |
| Wallet balance after session | SELECT w.balance, w.user_id FROM wallets w WHERE w.user_id = 33 |
| User RFID tags | SELECT id, ocpp_tag_id, tag_type, is_primary, is_active FROM user_tags WHERE app_user_id = 33 |
| SteVe tag activity | SELECT id_tag, active_transaction_count, in_transaction FROM stevedb.ocpp_tag_activity WHERE id_tag IN (SELECT id_tag FROM user_tags WHERE app_user_id=33) |
| Charger connector status | SELECT c.charge_box_id, c.connector_id, cs.status, cs.status_timestamp FROM stevedb.connector c JOIN stevedb.connector_status cs ON cs.connector_pk = c.connector_pk WHERE c.charge_box_id = "CS-HPC350K-00001" AND c.connector_id > 0 ORDER BY cs.status_timestamp DESC LIMIT 6 |
| Active pricing for all chargers | SELECT charge_box_id, pricing_model, rate_per_kwh, session_fee, display_name FROM charger_pricing WHERE is_active=1 ORDER BY charge_box_id |
| Steve sync queue status | SELECT action, status, attempts, last_error FROM steve_sync_queue ORDER BY created_at DESC LIMIT 10 |
| Webhook events (last 10) | SELECT event_type, charge_box_id, processed_at FROM webhook_events ORDER BY processed_at DESC LIMIT 10 |
| Charger online status | SELECT charge_box_id, last_heartbeat_timestamp, registration_status FROM stevedb.charge_box ORDER BY charge_box_id |

# Test Coverage Summary

| Module | Total TCs | P0/P1 | P2/P3 | Type |
| --- | --- | --- | --- | --- |
| 1 — Authentication | 8 | 8 | 0 | Auto + Manual |
| 2 — Map & Discovery | 14 | 10 | 4 | Auto + Manual |
| 3 — Charging Session | 21 | 18 | 3 | Auto + SAP CLI |
| 4 — Wallet | 6 | 5 | 1 | Auto + Manual |
| 5 — RFID & Tags | 7 | 6 | 1 | Auto + Manual |
| 6 — Reservations | 4 | 3 | 1 | Manual + CLI |
| 7 — Vehicles & Profile | 7 | 5 | 2 | Manual |
| 8 — Fleet Management | 5 | 3 | 2 | Manual |
| 9 — Push Notifications | 5 | 4 | 1 | Dev Build |
| 10 — OCPP 1.6J Compliance | 10 | 8 | 2 | Auto + CLI |
| 11 — Performance | 7 | 6 | 1 | Auto + Load |
| 12 — UI / UX | 10 | 7 | 3 | Manual |
| TOTAL | 104 | 83 | 21 |  |

WARNING P0/P1 tests must all pass before any production release. P2/P3 are tracked but non-blocking.