Operations Guide

Adding a New Charger

Version 1.0  |  May 2026  |  VoltStartEV Engineering

# Overview

This document describes the complete procedure for adding a new EV charging station to the VoltStartEV network. Follow every step in order — skipping steps will result in the charger appearing offline or failing to accept charging sessions.

The process updates three systems:

- SteVe OCPP Server — registers the charger for OCPP 1.6 communication

- VoltStartEV Database — adds charger metadata, capabilities, connector types, and pricing

- Mobile App — reflects changes automatically within 10 minutes via cache TTL

WARNING All commands must be run on the VoltStartEV backend server (136.113.7.146) as root or the deploy user.

# Prerequisites

|  | Requirement | Verify At |
| --- | --- | --- |
| ☐ | Charger physically installed and powered on | On-site inspection |
| ☐ | Network connection active (WiFi or SIM) | Ping the charger IP |
| ☐ | OCPP URL configured: ws://136.113.7.146:8080/steve/websocket/CentralSystemService | Charger web interface |
| ☐ | Charge Box ID format decided (see Section 3) | This document |
| ☐ | Number of connectors and connector types confirmed | Physical inspection |
| ☐ | Maximum power per connector known (kW) | Charger nameplate / datasheet |
| ☐ | GPS coordinates ready (6 decimal places) | Google Maps |
| ☐ | Pricing model decided (see companion Pricing Guide) | Operations team |
| ☐ | MySQL access to server available | SSH into 136.113.7.146 |

# Step 1 — Decide the Charge Box ID

Every charger needs a unique Charge Box ID. This ID is used in OCPP, the database, and the mobile app. It cannot be changed without disrupting active sessions.

### Naming Convention

Format: CS-{TYPE}{POWER}-{SEQUENCE} Examples: CS-AC7K-00003 7 kW AC charger, unit 3 CS-AC22K-00001 22 kW AC charger, unit 1 CS-DC50K-00002 50 kW DC charger, unit 2 CS-DC150K-00001 150 kW DC fast charger, unit 1 CS-HPC350K-00002 350 kW HPC ultra-fast, unit 2 CS-BHARAT-00001 Bharat AC/DC standard (2W/3W), unit 1 CS-GBT60K-00001 60 kW GB/T (BYD/Chinese EVs), unit 1

TIP Check existing IDs first: SELECT charge_box_id FROM charger_config ORDER BY charge_box_id;

### Type Code Reference

| Code | Max Power | Connector Type | Compatible Vehicles |
| --- | --- | --- | --- |
| AC7K | 7.2 kW | Type 2 / Bharat AC | Nexon EV, ZS EV, Atto 3, most 4W |
| AC11K | 11 kW | Type 2 | Tata Punch EV, premium 4W |
| AC22K | 22 kW | Type 2 | Tesla, Kia EV6, premium 4W |
| DC50K | 50 kW | CCS2 + CHAdeMO | All DC-capable 4W |
| DC150K | 150 kW | CCS2 | Nexon EV Max, Tata Punch EV |
| HPC350K | 350 kW | CCS2 | Kia EV6, BYD Atto 3, premium |
| BHARAT | 15 kW | Bharat AC-001 / DC-001 | 2-Wheeler, 3-Wheeler, E-Rickshaw |
| GBT | 60 kW | GB/T | BYD models, Chinese imports |

# Step 2 — Register in SteVe OCPP Server

SteVe handles the physical OCPP protocol. The charger must exist in SteVe before it can connect.

NOTE SteVe web interface: http://136.113.7.146:8080/steve — Login: admin / admin

### Option A — SteVe Web Interface (recommended)

- Open http://136.113.7.146:8080/steve → Charge Points → Add New

- Enter the Charge Box ID exactly as decided in Step 1

- Set Registration Status = Accepted

- Click Save

### Option B — Direct MySQL

mysql -u root -p stevedb -e " INSERT INTO charge_box ( charge_box_id, registration_status, insert_connector_status_after_transaction_msg ) VALUES ( 'CS-AC22K-00003', 'Accepted', 0 ); "

WARNING Physically restart the charger after registration so it sends a BootNotification to SteVe. The charger will not appear online until SteVe receives BootNotification.

# Step 3 — Add to App Database

The app database has two tables to update: charger_config (metadata + location) and charger_capabilities (per-connector detail including connector type and vehicle compatibility).

CAUTION charger_capabilities does not exist yet in the database. Run the CREATE TABLE in Step 3a before inserting data.

### 3a — Create charger_capabilities table (one-time setup)

mysql -u $APP_DB_USER -p"$APP_DB_PASSWORD" $APP_DB_NAME -e " CREATE TABLE IF NOT EXISTS charger_capabilities ( id INT UNSIGNED AUTO_INCREMENT PRIMARY KEY, charge_box_id VARCHAR(255) NOT NULL, connector_id INT NOT NULL DEFAULT 1, max_power_watts INT NOT NULL, power_type ENUM('AC','DC') NOT NULL DEFAULT 'AC', connector_type ENUM('Type2','CCS2','CHAdeMO','GBT','Type1', 'BharatAC','BharatDC','ThreePin') NOT NULL DEFAULT 'Type2', vehicle_category SET('2W','3W','4W','Bus','Truck') NOT NULL DEFAULT '4W', created_at DATETIME DEFAULT NOW(), PRIMARY KEY (id), UNIQUE KEY uq_charger_connector (charge_box_id, connector_id), INDEX idx_charge_box (charge_box_id) ); "

TIP After creating the table, also add location columns to charger_config (Step 3b).

### 3b — Add location columns to charger_config (one-time migration)

mysql -u $APP_DB_USER -p"$APP_DB_PASSWORD" $APP_DB_NAME -e " ALTER TABLE charger_config ADD COLUMN IF NOT EXISTS street VARCHAR(255) NULL, ADD COLUMN IF NOT EXISTS city VARCHAR(100) NULL, ADD COLUMN IF NOT EXISTS state VARCHAR(100) NULL, ADD COLUMN IF NOT EXISTS pincode VARCHAR(10) NULL, ADD COLUMN IF NOT EXISTS latitude DECIMAL(10,6) NULL, ADD COLUMN IF NOT EXISTS longitude DECIMAL(10,6) NULL, ADD COLUMN IF NOT EXISTS is_public TINYINT(1) DEFAULT 1, ADD COLUMN IF NOT EXISTS is_active TINYINT(1) DEFAULT 1; "

### 3c — Insert charger_config row

mysql -u $APP_DB_USER -p"$APP_DB_PASSWORD" $APP_DB_NAME -e " INSERT INTO charger_config ( charge_box_id, display_name, max_power_w, power_type, street, city, state, pincode, latitude, longitude, is_public, is_active ) VALUES ( 'CS-AC22K-00003', -- Must match SteVe charge_box_id exactly 'Koregaon Park Charger 3', -- Display name shown in app 22000, -- Max power in Watts (22 kW = 22000) 'AC_3_PHASE', -- AC_3_PHASE ¦ AC_1_PHASE ¦ DC '12, North Main Road', -- Street address 'Pune', 'Maharashtra', '411001', 18.537600, -- Latitude — 6 decimal places from Google Maps 73.893800, -- Longitude — 6 decimal places from Google Maps 1, -- 1 = public, 0 = private/restricted 1 -- 1 = visible in app, 0 = hidden ); "

### 3d — Insert charger_capabilities rows (one per connector)

Add one row per physical connector socket. A dual-port charger needs two INSERT statements with connector_id 1 and 2.

mysql -u $APP_DB_USER -p"$APP_DB_PASSWORD" $APP_DB_NAME -e " INSERT INTO charger_capabilities (charge_box_id, connector_id, max_power_watts, power_type, connector_type, vehicle_category) VALUES ('CS-AC22K-00003', 1, 22000, 'AC', 'Type2', '4W'), ('CS-AC22K-00003', 2, 22000, 'AC', 'Type2', '4W'); " -- For a Bharat AC/DC charger (2W + 3W compatible): INSERT INTO charger_capabilities (charge_box_id, connector_id, max_power_watts, power_type, connector_type, vehicle_category) VALUES ('CS-BHARAT-00001', 1, 7200, 'AC', 'BharatAC', '2W,3W'), ('CS-BHARAT-00001', 2, 15000, 'DC', 'BharatDC', '3W'); "

### Connector Type Quick Reference

| connector_type | Full Standard Name | power_type | vehicle_category values |
| --- | --- | --- | --- |
| Type2 | IEC 62196-2 (Mennekes) | AC | 4W |
| CCS2 | Combined Charging System 2 | DC | 4W |
| CHAdeMO | CHAdeMO DC standard | DC | 4W |
| GBT | GB/T 20234 (Chinese standard) | DC | 4W |
| Type1 | SAE J1772 Type 1 | AC | 4W |
| BharatAC | IS 17017-2-3 Bharat AC-001 | AC | 2W,3W |
| BharatDC | IS 17017-2-3 Bharat DC-001 | DC | 2W,3W |
| ThreePin | 3-Pin 5A/15A socket | AC | 2W,3W |

# Step 4 — Add Pricing

Every charger must have at least one active pricing record. Without pricing, the app shows "Contact operator for pricing details" and the cost estimate will not work.

TIP See the companion document VoltStartEV — Pricing Configuration Guide for all five pricing models. The quickest setup is shown below.

-- Flat per-kWh pricing (simplest setup): mysql -u $APP_DB_USER -p"$APP_DB_PASSWORD" $APP_DB_NAME -e " INSERT INTO charger_pricing (charge_box_id, pricing_model, rate_per_kwh, session_fee, display_name, valid_from, is_active) VALUES ('CS-AC22K-00003', 'per_kwh', 9.00, 0.00, '₹9.00/kWh', NOW(), 1); "

# Step 5 — Invalidate Cache and Verify

### 5a — Restart backend to clear cache

cd /build/VoltStartEV_Backend pm2 restart voltstartev # Verify the charger appears in the API: curl -s http://136.113.7.146:3000/api/chargers ¦ python3 -c \ "import sys,json; [print(c['chargeBoxId']) for c in json.load(sys.stdin)['data']]" \ ¦ grep CS-AC22K-00003

### 5b — Verify OCPP connection

mysql -u root -p stevedb -e " SELECT charge_box_id, last_heartbeat_timestamp, registration_status, ocpp_protocol FROM charge_box WHERE charge_box_id = 'CS-AC22K-00003'; " -- Expected: last_heartbeat within 2 minutes, status = Accepted mysql -u root -p stevedb -e " SELECT c.charge_box_id, c.connector_id, cs.status FROM connector c JOIN connector_status cs ON cs.connector_pk = c.connector_pk WHERE c.charge_box_id = 'CS-AC22K-00003'; " -- Expected: each connector shows status = Available

# Step 6 — End-to-End Test Checklist

|  | Test | Expected | Result |
| --- | --- | --- | --- |
| ☐ | Charger appears on map in VoltStartEV app | Marker visible |  |
| ☐ | Correct connector type shown in modal | e.g. Type 2 |  |
| ☐ | Pricing displayed before starting session | ₹X.XX/kWh |  |
| ☐ | Connector status shows Available | Green indicator |  |
| ☐ | Start session via app — session screen appears | kWh updating |  |
| ☐ | Stop session — session completes normally | Summary shown |  |
| ☐ | Session appears in History with correct cost | Cost matches |  |
| ☐ | Wallet balance reduced by correct amount | Balance matches |  |
| ☐ | Connector returns to Available after stop | Green indicator |  |

# Troubleshooting

| Symptom | Resolution |
| --- | --- |
| Charger not on app map | Check charger_config.is_active = 1. Restart backend to clear 10-min cache. |
| Connector shows Unavailable | Physically restart charger to trigger BootNotification to SteVe. |
| No pricing shown | Verify charger_pricing row has is_active = 1 and valid_from <= NOW(). |
| Session start fails | Check SteVe registration_status = Accepted. Confirm OCPP connection on port 8080. |
| charger_capabilities missing | Run the CREATE TABLE statement in Step 3a. |
| Latitude/longitude columns missing | Run the ALTER TABLE statement in Step 3b. |
| Heartbeat not updating | Check firewall — port 8080 must be open to charger IP. |
| Wrong vehicle types in filter | Update charger_capabilities.vehicle_category for the affected connectors. |

# Quick Reference — Minimum Required DB Records

stevedb.charge_box -- 1 row (OCPP registration) voltstartev_db.charger_config -- 1 row (location + metadata) voltstartev_db.charger_capabilities -- N rows (one per connector) voltstartev_db.charger_pricing -- 1+ rows (at least one active)